RESEARCH METHODOLOGY

Power Quality Improvement of Power System Using Artificial Intelligence (AI)

# 1. Overview of Proposed Methodology

The proposed research adopts a novel end-to-end deep learning pipeline for automated Power Quality Disturbance (PQD) detection and classification, implemented entirely in Python. The framework integrates signal preprocessing, deep transfer learning, and an explainability layer — addressing the critical gaps of low transparency and reliance on synthetic data identified in existing literature. The methodology is structured into six interconnected phases as described below.

# 2. Phase-Wise Methodology

## Phase 1: Data Acquisition and Real-World Dataset Construction

Unlike prior works that rely on synthetically generated waveforms, this research will construct a hybrid dataset combining:

- Real-world PQ recordings captured using calibrated power quality analyzers (e.g., Fluke 435-II) at industrial and residential feeders.

- IEEE 1159 and IEC 61000-4-30 compliant benchmark disturbances including voltage sags, swells, harmonics, flicker, transients, and interruptions.

- Augmented synthetic data generated using parametric equations with realistic noise profiles (SNR: 20–40 dB) using Python's NumPy and SciPy libraries.

All waveforms will be sampled at 12.8 kHz (256 samples/cycle at 50 Hz), labelled per IEC standard categories, and stored in HDF5 format using h5py for efficient large-scale access.

## Phase 2: Signal Preprocessing and Feature Extraction

Raw voltage/current signals will be processed using a multi-domain feature extraction pipeline:

- Time-Frequency Analysis: Discrete Wavelet Transform (DWT) using PyWavelets (pywt) with Daubechies-4 (db4) mother wavelet across 5 decomposition levels to capture transient and steady-state disturbances simultaneously.

- S-Transform: Applied via custom Python implementation to provide time-frequency localization superior to STFT for non-stationary PQ signals.

- Statistical Features: RMS, THD, crest factor, kurtosis, skewness extracted per window using SciPy.stats.

Features will be normalized using StandardScaler (scikit-learn) and dimensionality-reduced via Principal Component Analysis (PCA) retaining 95% variance — reducing input complexity while preserving discriminative information.

## Phase 3: Deep Transfer Learning Model Architecture

A novel Convolutional Neural Network with Transfer Learning backbone (ResNet-50 / EfficientNet-B3, pre-trained on ImageNet) will be adapted for PQD classification — a technique not yet widely explored in this domain. The adaptation involves:

- Converting 1-D waveform signals into 2-D scalogram images using Continuous Wavelet Transform (CWT) via the PyWavelets and Matplotlib libraries, enabling use of powerful image-classification architectures.

- Fine-tuning the final three convolutional blocks on the PQ dataset using TensorFlow 2.x / Keras with Adam optimizer (lr = 1e-4) and categorical cross-entropy loss.

- A custom classification head: GlobalAveragePooling → Dense(256, ReLU) → Dropout(0.4) → Dense(N_classes, Softmax), where N_classes covers 15 PQD categories per IEEE 1159.

To further capture temporal dependencies in multi-cycle disturbance sequences, a hybrid CNN-BiLSTM module will be appended, processing flattened CNN features through Bidirectional LSTM layers (128 units per direction) — combining spatial and temporal learning in a single end-to-end trainable architecture.

## Phase 4: Intelligent Harmonic Source Identification Using ANN

Extending the synopsis's ANN-based approach, a Multilayer Perceptron (MLP) with three hidden layers (512-256-128 neurons, ReLU activations) will be trained to model the input-output relationship between individual nonlinear load harmonic currents and the resulting voltage harmonic distortion at the point of common coupling (PCC). The Python implementation will use:

- scikit-learn's MLPRegressor for rapid prototyping and baseline benchmarking.

- A custom TensorFlow model for fine-grained control over weight initialization, regularization (L2, λ=0.001), and sensitivity analysis.

- Sensitivity-based harmonic contribution ranking: each load's harmonic current is perturbed by ΔI% = ±5% independently, and the ANN output voltage response quantifies the contribution — providing an interpretable ranking of harmonic polluters.

## Phase 5: Explainability and Transparency Layer (XAI)

Addressing the key barrier of low transparency identified in the literature review, Explainable AI (XAI) techniques will be integrated:

- Grad-CAM (Gradient-weighted Class Activation Mapping): Visualizes which time-frequency regions of the scalogram image most influenced the classification decision, implemented using the tf-keras-vis library.

- SHAP (SHapley Additive exPlanations): Quantifies feature importance for the ANN harmonic identifier using the shap Python library, providing regulatory-compliant decision explanations.

This phase directly addresses the novelty gap in existing PQ deep learning literature and makes the framework suitable for adoption by utility engineers who require interpretable decisions.

## Phase 6: Real-Time Deployment and Validation

The final phase involves deployment of the trained model in a simulated real-time environment:

- MATLAB/Simulink co-simulation via Python-MATLAB Engine API for hardware-in-loop (HIL) validation against IEEE 13-bus and 33-bus test systems.

- ONNX model export using tf2onnx for edge deployment on Raspberry Pi 4 / NVIDIA Jetson Nano, validating real-time feasibility (target inference latency < 50 ms per window).

- Performance benchmarking against state-of-the-art methods: classification accuracy, F1-score, confusion matrix, and processing time — evaluated under both clean and noisy (SNR = 20 dB) conditions using stratified 5-fold cross-validation.

# 3. Python Implementation Stack

The complete implementation will be open-source on GitHub and structured as a modular Python package:

Data & Signal: NumPy, SciPy, PyWavelets, h5py     ML & DL: TensorFlow 2.x, Keras, scikit-learn, PyTorch (baseline comparison)

XAI: SHAP, tf-keras-vis (Grad-CAM)     Visualization: Matplotlib, Seaborn, Plotly     Deployment: ONNX, TFLite, FastAPI (REST inference server)

# 4. Novelty and Contribution

- First application of CWT-scalogram + pre-trained CNN Transfer Learning to PQ disturbance classification using real-world (non-synthetic) Indian grid data.

- Hybrid CNN-BiLSTM architecture for simultaneous spatial and temporal feature learning — novel in PQ domain.

- Integration of SHAP and Grad-CAM XAI techniques into a PQ monitoring framework, providing regulatory-compliant transparent decisions.

- Sensitivity-based harmonic source ranking using MLP ANN, validated on a multi-load industrial feeder test case.

- Real-time edge deployment validation on embedded hardware — bridging the gap between academic models and practical utility deployment.

Department of Electrical Engineering, BIT Sindri, Dhanbad — PhD Research Synopsis Methodology | 2024